%% Section 1: Load the indoor scene

viewer = siteviewer(SceneModel="office.stl", ShowOrigin=false, Transparency=0.25);

%% Section 2: Place transmitter and receivers

tx = txsite("cartesian", AntennaPosition=[2; 1.3; 2.5]);

rx_desk = rxsite("cartesian", AntennaPosition=[3.6; 7.5; 1]);

rx_shelf = rxsite("cartesian", AntennaPosition=[0.4; 3.3; 1]);

show(tx)

show(rx_desk)

show(rx_shelf)

los(tx, [rx_desk rx_shelf])

pm = propagationModel("raytracing", CoordinateSystem="cartesian", SurfaceMaterial="wood");

reflectionLevels = 0:3;

receivedPower_desk = zeros(size(reflectionLevels));

receivedPower_shelf = zeros(size(reflectionLevels));

for i = 1:length(reflectionLevels)

pm.MaxNumReflections = reflectionLevels(i);

receivedPower_desk(i) = sigstrength(rx_desk, tx, pm);

receivedPower_shelf(i) = sigstrength(rx_shelf, tx, pm);

fprintf("Reflections=%d -> Desk=%.2f dBm | Shelf=%.2f dBm\n", ...

reflectionLevels(i), receivedPower_desk(i), receivedPower_shelf(i));

end

figure

plot(reflectionLevels, receivedPower_desk, '-o', 'DisplayName', 'Desk (blocked)')

hold on

plot(reflectionLevels, receivedPower_shelf, '-s', 'DisplayName', 'Shelf (line of sight)')

hold off

xlabel('Max Number of Reflections')

ylabel('Received Power (dBm)')

title('Effect of Reflections on Received Power')

legend

grid on

pm = propagationModel("raytracing", CoordinateSystem="cartesian", SurfaceMaterial="wood");

% --- Case 1: Reflection only ---

pm.MaxNumReflections = 1;

pm.MaxNumDiffractions = 0;

clearMap(viewer)

raytrace(tx, rx_desk, pm)

ss_reflectionOnly = sigstrength(rx_desk, tx, pm);

fprintf("Reflection only  -> %.2f dBm\n", ss_reflectionOnly);

% --- Case 2: Diffraction only ---

pm.MaxNumReflections = 0;

pm.MaxNumDiffractions = 1;

clearMap(viewer)

raytrace(tx, rx_desk, pm)

ss_diffractionOnly = sigstrength(rx_desk, tx, pm);

fprintf("Diffraction only -> %.2f dBm\n", ss_diffractionOnly);

% --- Case 3: Combined ---

pm.MaxNumReflections = 1;

pm.MaxNumDiffractions = 1;

clearMap(viewer)

raytrace(tx, rx_desk, pm)

ss_combined = sigstrength(rx_desk, tx, pm);

fprintf("Combined         -> %.2f dBm\n", ss_combined);

%% Section 7: Bar chart comparing the three cases

figure

bar(categorical({'Reflection Only','Diffraction Only','Combined'}), ...

[ss_reflectionOnly, ss_diffractionOnly, ss_combined])

ylabel('Received Power (dBm)')

title('Desk Receiver: Reflection vs Diffraction vs Combined')

grid on